OIL

Source

- Cargo heating coils.

- Slop tank heating coils.

- Bunker heating.

- Fuel oil heaters.

- Lubrication oil heating coils.

- Purifier heaters.

Effects

- Combines with suspended solids in the boiler water to produce sticky deposits.

- Adheres to boiler tubes thus reducing heat transfer and hence increase the chance of tube failure.

- Causes foaming of boiler water resulting in carryover of oil and boiler water solids.

Prevention

- Look for oil traces in feed filters.

- Periodically carry out hydraulic test on heating coils.

- Check observation tank for fuel oil, and clean out if observed.

- Take great care when dumping oil slops in engine room.

- Check if there is any oil in the deaerator.