CHLORIDES

Source

- Carryover of sea water droplets from evaporator.

- Leakage of sea water into condensate system.

- Use of untreated fresh water as boiler make up.

Effects

- Corrosion of boiler surfaces due to high chloride ion concentration and possible formation of hydrochloric acid in the boiler.

- Loss of alkalinity and pH.

Prevention

- Minimise the carryover of sea water from evaporator.

- Stop leaks of sea water to condensate system.

- Do not use untreated fresh water as boiler make-up water.

- Maintain recommended level of alkalinity in boiler water.

- Do not exceed recommended level of chloride in the boiler water.