MAGNETIC REED SWITCH

Range :

- Point of contact only

Activated by :

- Direct displacement between door and frame/window and frame of not more than 4 in. (100mm); 1-2 in (25-50mm) normal

Uses :

- Primarily door and window openings, but also cabinet, drawers, etc.

Typical dimensions :

- Flush circular, typically 0.4 in. (11mm) diameter x 1.3 in (32mm) deep

Associated work :

- Best installation have conduit to door/window frame fully concealed

Advantages :

- Extensively uses, easily understood, reliable

Disadvantages :

- Will not detect cutting through door or window material to affect entry