WEATHER & SPACE QUALITY SENSOR

Sensor specification (Solar radiation )

PANEL  X-RAY

-Flat panel sensors with specifications optimized for measurement applications such as digital radiography and diffraction.

### Specifications

| Active Area | 124.8 x 124.8 mm |
| --- | --- |
| Pixel Size | 200 x 200 (Fast/Partial mode), 100 x 100 (Fine/Panoramic mode) um |
| Frame Rate |  |
| Resolution | 2.5 line pairs/mm |
| Dynamic Range |  |
| Scintillator | CsI |
| Digital Output | 13 bit |
| Note |  |
| Measurement Condition | Typ. Ta=25 deg. C, unless otherwise noted |

pyranometer

Pyranometer is designed for field measurement of global solar radiation in agricultural, meteorological,and solar energy studies.

LII--220000SA SPPEECCIIFFIICCAATTIIONNSS

Calibration: Calibrated against an Eppley

Precision Spectral Pyranometer (PSP) under natural daylight conditions. Typical error under these conditions is ± 5%.

Sensitivity: Typically 90 μA per 1000 W m-2.

Linearity: Maximum deviation of 1% up to

3000 W m-2.

Stability: < ± 2% change over a 1 year period.

Response Time: 10 μs.

Temperature Dependence: 0.15% per °C

maximum.

Cosine Correction: Cosine corrected up to 80° angle of incidence.

Azimuth: < ± 1% error over 360° at 45° elevation.

Tilt: No error induced from orientation.

Operating Temperature: -40 to 65°C

\ Relative Humidity: 0 to 100%.

Detector: High stability silicon photovoltaic

detector (blue enhanced).

Sensor Housing: Weatherproof anodized aluminum case with acrylic diffuser and stainless steel hardware.

Size: 2.38 Dia. 2.54 cm H (0.94” 1.0”).

Weight: 28 g (1 oz).

Cable Length: 3.0 m (10 ft).

# 240-8104 Albedometer

- albedometers are suitable for measuring global radiation and/or albedo over many differing types of surface.

## Specifications

Sensing element: 12 black & white copper segments with 6 thermocouples each
Sensitivity: 0.3 to 3 µm
Azimuth response: < 3% of the value
Cosine response: < 3% 0.95% of the value, 0° to 80° zenith angle
  ± 3% from 60° to 80° zenith angle
Response time: < 25 seconds
Measuring range: 0-1500 Wm²
Resolution: < 1 Wm²
Stability: < 1% per year (temporary operation)
Temperature effect: < 1% of the value between -20 °C to 40°C
Linearity: < 0.5% in the range 0.5-1330 W/m²
Impedance: Approx 35 ohms
Ambient temperature: -40°C to +60°C
Windshield: Polished glass dome, 2.75" dia x 0.08" thick (70 x 2 mm)
Leveling: Bull's-eye level on each face
Size: 14.6" L x 4.5" W x 4.2" H (371 x 114 x 107 mm)
Weight/shipping: 3.5 lbs/10 lbs (1.6 kg/4.5 kg)
Cable: 2 polar shielded, 3 m length

# PHOTOSYNTHETICALLY ACTIVE RADIATION (P.A.R) SENSOR - JYP 1000

# TECHNICAL SPECIFICATIONS

Height : 26 mm. 

Diameter : 22 mm. 

Material : Polycarbonate 

Diffuser : Acrylic Type. 

Blue filter : SCHOTT glass tinted from the block. 

Sensor fixation system : by stainless steel screw situated under the sensor (screw : 1/4) as photographic device. 

Output cable : Two cables with screen. 

Lenght cable : 2 meters in standard version. Longer cables are available (contact us). 

Charge resistor : The value depends on factory calibration. The average value is about 220ohm. 

Output level : 10 mV for 536 µmol.m-2.s-1. 
Each sensor is delivered with an individual 
calibration certificate. 

Temperature sensibility : The effect of temperature on the sensor is very low (better than 0,1%/°C at +20°Celsius). 

Stability : JYP 1000 sensor is remarquably stable.

pyrgeometer

## PRODUCT SPECIFICATION

| Spectral range | 4500 to 42000 nm |
| --- | --- |
| Sensitivity | 5 to 7 µV/W/m² |
| Response time | < 18 s |
| Window heating offset | < 15 W/m² |
| Zero offset B | < 4 W/m² |
| Temperature dependence of sensitivity (-10 ºC to +40 ºC) | < 5 % |
| Operating temperature range | -40 to +80 °C |
| Net irradiance range | -250 to +250 W/m² |
| Field of view | 150 ° |
| Non-linearity | < 1 |

Photosynthetically Active Radiation (PAR) Sensor

Photosynthetically Active Radiation (PAR) is the spectral range of solar light from 400 to 700 nanometers that is used by aquatic plants and algae in photosynthesis.

Why measure PAR?

• Measurement of light attenuation

• Primary productivity studies

• Development of Photosynthesis-Irradiance (P-I) curves

• Photoinhibition studies

• Submerged Aquatic Vegetation (SAV) studies

• Measurement of the euphotic zone

• Investigate absorbing material such as DOM, pigments, or gelbstoff

• Measurement of Photosynthetic Photon Flux Density (PPFD)

| PAR Sensor Specifications |  |
| --- | --- |
| Maximum operating depthnsSpecifications | P100 ftAR Sensor Specifications |
| CompatibilitySpecifications | New or existing YSI 6600*, 6600EDS, or 6600V2 sondesR Sensor Specifications |
| Measurementsr Specifications | Averaged over a 60-second time period, filter time user-adjustableR Sensor Specifications |
| Unitsnsor Specifications | Millivolts or μmoles/sec/m2R Sensor Specifications |
| Absolute calibration Sensor Specifications | PAR S±5%ensor Specifications |
| SensitivitySensor Specifications | Typically 3 μA per 1000 μmol s-1 m-2 in waterSensor Specifications |
| Linearity Sensor Specifications | PMaximum deviation of 1%nsor Specifications |
| Stabilitysor Specifications | <±2% change over one-year periodecifications |
| Response timesor Specifications | PAR10 μs Sensor Specifications |
| PAR systems Sensor Specifications | PAUnattended monitoring: Upper and lower stainless steel mounting brackets, sensor mounting arm bracket set for 0.5 meter sensor separation, 2 PAR sensor electronics, 2 five-pin wet-pluggable PAR connectors with dummy plugs Manual profiling: Stainless steel mounting brackets, sensor/counterbalance mounting arms and collapsible brackets, 2 channel PAR sensor electronics, 2 PAR connectors with dummy plugsR Sensor Specifications |

Pyrgeometers

A pyrgeometer provides a voltage that is proportional to the radiation exchange between the instrument and the sky (or ground) in its field of view. The detector signal output can be positive or negative.

| Specifications | CGR 3 | CGR 4 |
| --- | --- | --- |
| Response time (95 %) | < 18 s | < 18 s |
| Non-stability (change/year) | < 1 % | < 1 % |
| Non-linearity (-250 to 250 W/m²) | < 1 % | < 1 % |
| Window heating offset (with 1000 W/m² solar radiation) | < 15 W/m² | < 4 W/m² |
| Temperature dependence of sensitivity | < 5 % (-10 °C to +40 °C | < 1 % (-20 °C to +50 °C) |
| Sensitivity | 5 to 7 μV/W/m² | 5 to 10 μV/W/m² |
| Operating temperature | -40 °C to +80 °C | -40 °C to +80 °C |
| Field of view | 150 ° | 180 ° |
| Spectral range (50 % points) | 4.5 to 42 _m | 4.5 to 42 _m |
| Irradiance (net) | -250 to 250 W/m² | -250 to 250 W/m² |
| CGR instruments have a standard cable length of 10 m. Optional cable lengths 25 m and 50 m |  |  |
| Standard 10 K Thermistor (YSI 44031) or optional Pt-100 temperature sensor |  |  |
| Under most conditions the output from CGR pyrgeometers is negative and suitable data acquistion equipment must be use |  |  |

Radiometer

Radiometers are used to measure the intensity of electromagnetic radiation (e.g., UVC, UVB, UVA, Vis, and IR). They are univariate devices that do no distinguish between light of different wavelengths. In addition to wavelength range, radiometers carry specifications such as sensor diameter, power range, accuracy, resolution, operating temperature range, and humidity range.  The wavelength range specifies the detection range for radiometers. The sensor diameter measures the active part of the sensor. Power range is the power per unit area. Radiometer accuracy is usually given as a percentage. The resolution of the detector is a measure of radiometer sensitivity. Operating temperature and humidity range are also important specifications to consider when selecting radiometers.

Radiometers Specifications

| Wavelength Range: |  | The wavelength range for the radiometer. |
| --- | --- | --- |
|  | Search Logic: |  | User may specify either, both, or neither of the limits in a "From - To" range; when both are specified, matching products will cover entire range. Products returned as matches will meet all specified criteria. |
|  | Sensor Diameter: |  | The diameter of the active part of the sensor. |
|  | Search Logic: |  | User may specify either, both, or neither of the "At Least" and "No More Than" values. Products returned as matches will meet all specified criteria. |
|  | Power Range: |  | The power per unit area range of the radiometer. |
|  | Search Logic: |  | User may specify either, both, or neither of the limits in a "From - To" range; when both are specified, matching products will cover entire range. Products returned as matches will meet all specified criteria. |
|  | Accuracy: |  | The accuracy of the radiometer. |
|  | Search Logic: |  | All matching products will have a value less than or equal to the specified value. |
|  | Resolution: |  | The resolution of the radiometer. |
|  | Search Logic: |  | All matching products will have a value less than or equal to the specified value. |
|  | Operating Temperature Range: |  | Operating temperature range is the recommended range for the radiometer. |
|  | Search Logic: |  | User may specify either, both, or neither of the limits in a "From - To" range; when both are specified, matching products will cover entire range. Products returned as matches will meet all specified criteria. |
|  | Humidity Range: |  | The operating humidity range for the radiometer. |
|  | Search Logic: |  | User may specify either, both, or neither of the limits in a "From - To" range; when both are specified, matching products will cover entire range. Products returned as matches will meet all specified criteria. |

Geiger Muller Counter